SMART SOLUTIONS FOR ENERGY & GRID  ·  PROBLEM STATEMENT

AI-Powered Renewable Generation Forecasting Platform

Forecasting What the Grid

Is About to Get

Probabilistic 24–72 hour solar and wind forecasting, with a decision layer that turns every forecast into a sized, priced and confidence-rated grid action.

| TEAM NAME | ZERO BIAS |
| --- | --- |
| TEAM LEAD | KRISH PARMAR |
| THEME | Renewable energy forecasting · Grid stability · Decision support |
| APPROACH | Physics-informed hybrid forecasting with calibrated uncertainty |
| VALIDATION | Belgium (Elia, metered) → India (transfer demonstration) |

Technical Proposal  ·  September 2026

# Executive Summary

Solar and wind output swings with the weather. Grid operators absorb that uncertainty by holding inflexible fossil generation in reserve — or by curtailing renewables outright. India curtailed 2.1 TWh of renewable electricity in FY 2025-26, and from 1 April 2026 the permitted forecast deviation for solar was halved from ±10% to ±5%. The cost of getting a forecast wrong roughly doubled this financial year.

We forecast regional solar and wind generation 24 to 72 hours ahead, and convert each forecast into a grid action.

## Three ideas carry the design

|  | Idea | Why it matters |
| --- | --- | --- |
| 1 | Physics does the deterministic work | A solar plant’s output at 11:00 on 14 March is ~90% determined by sun geometry and clear-sky radiative transfer — exactly computable today, with no training data. We compute it. Machine learning corrects only the residual: soiling, wake losses, local microclimate. |
| 2 | Every forecast is a verified interval | Point forecasts cannot size a reserve. We emit P10/P50/P90 and apply conformal calibration so the 80% band demonstrably covers 80% of outcomes — and we publish the coverage plot that proves it. |
| 3 | The output is an action, not a chart | Operators do not need a curve. They need to know that Tuesday 11:00–14:00 will be 340 MW over-generating with 80% confidence, that the battery absorbs 210 MWh, and what the remainder costs to curtail. |

## What this buys

| Property | Consequence |
| --- | --- |
| Low data requirement | The residual is small and structured; the raw signal is large and seasonal. |
| Cold start | A region with zero generation history gets a usable forecast on day one, from site parameters alone. |
| Interpretability | SHAP on the residual answers “why is this forecast low?” with low cloud and soiling — not an opaque attribution. |
| Graceful degradation | If the learned model drifts or fails, the system falls back to physics, not to nothing. |
| Transferability | Moving to a new grid changes configuration — not the model architecture. |

VALIDATION STRATEGY We validate on Belgium, where metered 15-minute data and the grid operator’s own published day-ahead forecast both exist — so our accuracy claim is measured against a professional operational system rather than a baseline we chose ourselves. We then transfer the identical pipeline to Indian data to show it moves to a new grid in hours. Belgium carries the accuracy claim; India carries the relevance claim. Neither is asked to do the other’s job.

# 1   Understanding the Problem

“Forecast renewable output” is not one problem. It is five, and they fail in different ways. Naming them separately is what prevents a team from building one monolithic model and calling it a platform.

| Sub-problem | The real question | Why it is hard |
| --- | --- | --- |
| Weather → resource | Turn a weather prediction into irradiance or hub-height wind at this location | NWP grids are 9–25 km. The error is systematic bias, not noise. |
| Resource → power | Turn irradiance or wind speed into megawatts | Strongly non-linear: inverter clipping, soiling, wake losses, storm cut-out. |
| Uncertainty | How wrong might this be, at each hour? | Error is heteroscedastic — near zero at 03:00, very large on a broken-cloud afternoon. |
| Event detection | Where are the ramps and over-generation windows? | A 2% RMSE gain is worth less than catching one 40% ramp. |
| Action | Curtail, charge, discharge or commit backup — and why | Needs demand, must-run capacity, storage state and ramp limits. |

## The horizon decides the method

Forecast skill is governed by which information remains useful at a given lead time. The brief specifies 24–72 hours — which lands squarely in the band where numerical weather prediction is the only meaningful signal.

| Horizon | Dominant signal | Past output helps? | Approach |
| --- | --- | --- | --- |
| 0–30 min | Satellite cloud motion | Strongly | Persistence + advection |
| 30 min – 6 h | Satellite, recent output | Usefully | Smart persistence, boosting on lags |
| 6–72 hours | Weather forecast only | Barely | NWP post-processing |
| 3–14 days | Ensemble NWP, climatology | No | Probabilistic ensemble |

THE CONSEQUENCE MOST TEAMS MISS At lead hour 48, yesterday’s last meter reading carries almost nothing. Any model trained on short autoregressive lags will score brilliantly in a naive backtest and fail completely in deployment, because the backtest leaked information the live system will never have. This reframes the project as NWP post-processing rather than time-series forecasting — the single most consequential decision in this proposal.

## The problem, in measured quantities

| Measure | Value | Source |
| --- | --- | --- |
| Renewable electricity curtailed in India, FY 2025-26 | 2.1 TWh | Ember, June 2026 |
| As a share of total renewable generation | 1.3% | Ember |
| Peak-hour curtailment of solar and wind, by April 2026 | 4% | Ember |
| Lost to transmission constraints, Q1 2026 alone | 300 GWh | Ember, 2026 |
| Lost on the single worst day, 30 March 2026 | 34 GWh | Ember |
| Storage India needs to prevent current curtailment | ~10 GWh | Ember estimate |
| Permitted solar deviation band, from 1 April 2026 | ±10% → ±5% | CERC DSM |
| Permitted wind deviation band, from 1 April 2026 | ±15% → ±10% | CERC DSM |

34 GWh lost in one day is roughly the daily electricity consumption of five million urban middle-class households. Meanwhile the CERC tolerance mechanism phases to zero by FY 2031, so the financial consequence of forecast error rises every year on a published schedule.

# 2   Proposed Solution

The core mechanism is a hybrid estimator. A physics engine converts weather to power deterministically using site parameters. A gradient-boosting model never has to learn that job — it sees only what physics got wrong, and predicts that residual at three quantiles.

Figure 1  Physics-informed residual learning — the mechanism that earns the accuracy

Remove the physics block and the machine-learning model must re-derive orbital mechanics from a few months of data. Keep it, and the model has a small, structured target: soiling, wake losses, sensor drift, local microclimate.

## Why not simply train a deep network end-to-end?

| Objection | Response |
| --- | --- |
| “Why not an LSTM?” | At 24–72 hour lead, autoregressive lags carry almost no information. This is NWP post-processing, not time-series forecasting. GEFCom2014 — the reference academic competition on exactly this problem — was won by a tree ensemble. |
| “Isn’t physics too rigid?” | Physics handles only what is genuinely deterministic: sun position, clear-sky radiative transfer, panel geometry, the turbine power curve. Everything atmospheric and asset-specific is learned. |
| “Does deep learning get benchmarked?” | Yes — as a documented experiment in the final phase. A benchmark showing TFT and N-HiTS losing to gradient boosting on our data is a stronger result than a marginal win, because it proves we measured rather than assumed. |

# 3   Technical Approach

## 3.1  Data strategy

Three inputs are required and rarely ship together: regional generation history, matching weather, and site parameters. One portal closes the loop — Elia publishes generation, its own forecast, and demand on a single 15-minute timebase with one join key.

| Role | Source | What it gives |
| --- | --- | --- |
| Solar & wind truth | Elia ods031/032, ods086/087 | Measured MW, monitored capacity, and the operator’s own day-ahead, week-ahead and P10/P90 forecasts |
| Demand | Elia ods001/002/003 | Measured and forecast total load — this is what makes the decision layer real rather than synthetic |
| Weather | Open-Meteo (ECMWF, ICON, GFS) | Identical variable names historically and forward, so training and serving cannot diverge. No API key. |
| Fleet parameters | site_master.yaml (authored) | Archetype rows: tilt, azimuth, hub height, power curve, capacity share |
| Transfer region | Zenodo 10.5281/zenodo.7824872 | Hourly Indian solar and wind capacity factors, state-level and gridded, 1979–2022, plus installed capacity |

Every input is open, free and licence-clear. Nothing in the critical path requires a paid subscription, a commercial key or a data-sharing agreement — which means the result is reproducible by anyone, including an evaluator.

## 3.2  Regional scale changes the physics layer

A region has no single tilt or hub height, so two adaptations are required:

- Fleet archetypes. site_master becomes a weighted set of representative configurations — for Belgian PV, roughly rooftop-south 35°, rooftop east-west 20° and ground-mount 30°. The physics chain runs per archetype and is summed by capacity share. This is what reproduces the morning shoulder that east-west rooftop arrays produce and a single-tilt model underestimates every day of the year.

- Capacity-weighted weather grid. Weather is sampled at 5–10 coordinates across the region, each weighted by nearby installed capacity. A single centroid systematically misses frontal systems crossing the region. Spatial aggregation is also why regional forecasting is easier than plant forecasting — independent cloud noise partially cancels.

## 3.3  Feature families

| Family | Representative features | Why it earns its place |
| --- | --- | --- |
| Solar physics | clearsky_index_kt, poa_global, cell_temperature, thermal_derate, physics_pac_mw, clipping_headroom | Removes the deterministic 90%. The clear-sky index makes January and June comparable. |
| Wind physics | ws_hub, air_density, ws_density_corrected, power_curve_cf, dP_dv, cut-in/cut-out flags | Power goes as v³ — a 10% wind-speed error is a 33% power error. Density correction alone recovers several points of nRMSE. |
| NWP quality | lead_hours, model_disagreement, ensemble_spread, nwp_bias_lag_7d, cloud low/mid/high split | These features predict our own error. They are what turns a point forecast into an honest interval. |
| Temporal | hour_sin/cos, doy_sin/cos, mins_since_sunrise, holiday flags | Cheap. Captures residual diurnal patterns physics misses — fog burn-off, sea breeze, dust. |
| Autoregressive | power_lag_24h, power_lag_168h, rolling statistics | Slow state only: soiling build-up, partial outages. Lags under 24 hours are excluded as leakage. |

THREE DATA TRAPS WE HANDLE BY NAME Curtailed hours are censored labels — measured output is not what the weather allowed, so training on them teaches the model to under-forecast exactly during the events we exist to predict; we set sample_weight to zero there. Night rows are free accuracy — half a solar series is zeros, so we report daylight-only metrics. Random train/test splits leak, because adjacent hours are near-identical; all evaluation is walk-forward with a gap.

## 3.4  Model ladder

Each rung must beat the one below it on a held-out window, or be skipped with a stated reason.

| # | Method | Role |
| --- | --- | --- |
| 0 | Persistence & climatology | The floor. Every reported number becomes a skill score against this. Non-negotiable. |
| 1 | Pure physics | Second baseline, later a feature. Works with zero history — this is the cold-start answer. |
| 2 | Gradient boosting (LightGBM) | The workhorse. Seconds to train, handles missing values natively, explainable via SHAP. |
| 3 | Residual learning | Model predicts the physics error, not the raw target. Largest single accuracy gain for the least code. |
| 4 | Quantile regression | Three heads at α = 0.1 / 0.5 / 0.9. Produces the interval the decision layer consumes. |
| 5 | Conformal calibration | Distribution-free finite-sample coverage guarantee. Cheap, and almost nobody does it. |
| 6 | Deep sequence models | N-HiTS, Temporal Fusion Transformer — benchmarked as a documented experiment, not assumed to win. |

## 3.5  Uncertainty, in two stages

- Quantile heads. Three LightGBM models with pinball-loss objectives. This produces intervals whose width varies with conditions — narrow at 03:00, wide on a broken-cloud afternoon. Width is driven by lead time, model disagreement, low cloud, and the power-curve gradient.

- Split-conformal calibration. On a rolling recent window, nonconformity scores adjust the interval so empirical coverage matches nominal coverage — regardless of whether the underlying model is well-specified. We report PICP and publish the reliability diagram.

An 80% band that covers 55% of outcomes is not a conservative forecast. It is a false statement, and it will size reserves wrong.

# 4   System Architecture

Figure 2  Five-layer platform architecture. The dashed loop is what makes it a platform rather than a notebook.

Measured actuals retrain the residual model nightly and recalibrate the intervals; operator decisions become labelled data. Training and inference call the identical feature code — which structurally removes the most common cause of deployed-model failure.

## Module map

| Module | Responsibility | Detail |
| --- | --- | --- |
| M1 | Ingestion | Source adapters, scheduling, unit and timezone normalisation at the boundary |
| M2 | Quality | Range checks, frozen-sensor detection, gap filling, curtailment masking |
| M3 | Feature engine ★ | pvlib chain, shear extrapolation, density correction, power curves, cyclic encoding |
| M4 | Forecast engine ★ | Baselines, physics model, residual GBDT, multi-horizon fit, model registry |
| M5 | Uncertainty ★ | Quantile heads, conformal calibration, coverage reporting |
| M6 | Event scanner | Ramp, over-generation and storm-shutdown detection with lead time and severity |
| M7 | Decision engine ★ | Net-load balance, storage simulation, ranked and priced actions |
| M8 | Backtesting | Walk-forward evaluation, skill scores, drift detection, SHAP attribution |
| M9 | API | /forecast, /outlook, /events, /actions, /backtest — versioned, with provenance |
| M10 | Dashboard | Fan chart, net-load stack, event timeline, action queue, regional map |

★ marks the critical path. Three interface contracts are fixed on day one — build_features, predict and recommend — so the team can work in parallel without collisions.

## Decision engine

Two derived quantities drive everything. Net load is demand minus forecast renewables; headroom is net load minus must-run capacity. Negative headroom means the grid physically cannot absorb the renewable output.

Figure 3  Forecast to action. The P10–P90 band is carried through, so each action inherits the forecast’s confidence.

## The virtual battery — a constraint turned into an output

Neither region publishes a usable grid-scale storage dispatch series, so storage is declared in configuration and simulated: energy capacity, power rating, round-trip efficiency, state-of-charge bounds and response time. This is the honest option, and it is more useful than real data would have been — because a configurable battery lets us sweep capacity from 0 to 1000 MWh and answer a question the sector is actively asking: what size battery would this region need?

## Technology stack

| Layer | Choice | Why |
| --- | --- | --- |
| Ingestion & validation | Python, httpx, APScheduler, pandera | Declarative schema contracts that fail loudly at every layer boundary |
| Storage | Parquet + DuckDB | Columnar, fast, zero-ops; TimescaleDB behind the same interface for production |
| Physics | pvlib-python, windpowerlib, IEC 61400-12 | Peer-reviewed reference implementations, not hand-rolled approximations |
| Models | LightGBM (quantile), MAPIE, SHAP, MLflow | Seconds to train, calibrated, explainable, version-tracked |
| Serving | FastAPI, Pydantic, React, Recharts/D3 | Typed contracts with automatic API documentation |

DEPLOYMENT REALITY The entire stack is open source — no licence cost, no vendor dependency in the critical path. It trains and serves on a laptop with no GPU. That is not just convenient: a system running on commodity hardware can be deployed at a state load dispatch centre without a procurement cycle.

# 5   Visualisation and Expected Results

The operator-facing output. The upper panel shows solar and wind forecasts with their P10–P90 bands against measured actuals; the lower panel converts them to net load and flags where the grid cannot absorb what is coming.

Figure 4  A 72-hour probabilistic outlook. The flagged window is an over-generation event detected two days ahead — charge the battery, then curtail the remainder.

Three things an evaluator should look for in this view: the band widens with lead time and with cloud, because uncertainty is not constant; the actuals stay inside the band, which is what calibration guarantees; and the over-generation window carries a megawatt-hour figure, not just a colour.

Figure 5  Left: forecast skill against published international benchmarks. Right: the storage sizing study — a planning output, not just an operational one.

The left panel is the chart that demonstrates understanding. A single 72-hour-average error number hides everything; the error-versus-lead-hour curve shows where skill actually comes from, and shows all four lines — persistence, physics-only, our model, and the operator’s own published forecast.

## Accuracy targets, and why they are credible

| Metric | Target | Basis |
| --- | --- | --- |
| Regional solar, day-ahead nRMSE (daylight-only) | 8–13% | Published range for regional day-ahead forecasts is 10–20% of installed capacity |
| Regional wind, day-ahead nRMSE | 10–15% | Consistent with the 7–19% single-farm MAE band, aggregated |
| Skill score versus persistence | > 0.40 | Persistence is very weak at 24–72 hours |
| Gap to Elia’s published day-ahead forecast | within 10–25% | Elia is a professional system with proprietary inputs; parity is not the goal |
| Coverage of the nominal 80% band (PICP) | 0.78–0.82 | Conformal calibration provides a finite-sample guarantee |

These targets are deliberately conservative and falsifiable. A target stated inside a published range, with the range cited, survives questioning. An impressive round number with no provenance does not.

## Evaluation protocol — decided before any model is trained

| Rule | Reason |
| --- | --- |
| Walk-forward splits, never random | Adjacent hours are near-identical; a random split leaks and inflates every number. |
| Metrics reported per lead hour | A 72-hour average hides where the skill is. The curve is the evidence. |
| Daylight-only for solar | Half a solar series is zeros. Including them halves the error and means nothing. |
| Normalised by installed capacity | Comparable across sites and regions of any size. |
| Matched information cutoff | Elia’s forecast is issued at a fixed time. Comparing against a different issue time is meaningless. |
| Curtailed intervals excluded | They are censored labels, not weather outcomes. |

# 6   Feasibility and Viability

## 6.1  Can the data be obtained?

Yes — and this was resolved before any modelling was designed. Every critical input is open, free, licence-clear and confirmed accessible. The one material gap is stated rather than hidden: India does not publish an open, sub-hourly, metered regional renewable series. That is precisely why the two-region design exists.

## 6.2  Can the accuracy be achieved?

| Published benchmark | Value |
| --- | --- |
| Day-ahead RMSE, regional aggregates | 10–20% of installed capacity |
| Single wind farm MAE, terrain-dependent | 7–19% |
| Mexico — wind: operators versus professional providers | 17% vs 13% MAE |
| Mexico — solar: operators versus professional providers | 9% vs 6% MAE |
| Share of world TSOs using centralised regional forecasting | ~80% |
| Optimal NWP combination | 5–6 models; minimum one global + one regional |

Two findings shape our design directly. Error grows nearly linearly with horizon, which validates treating lead time as a first-class feature. And combination forecasts beat single models — our three-model ingest sits at the practical floor of the recommendation. The market-structure finding matters too: a regional, centralised, multi-asset platform is the configuration the sector has converged on.

## 6.3  Can it be operated?

| Dimension | Assessment | Evidence |
| --- | --- | --- |
| Compute | Trivial | Full model set trains in under 5 minutes on one CPU core. Inference under 1 second. |
| Operating cost | Negligible | One small VM. Entire stack open source. |
| Steady-state effort | Near-unattended | Automated ingestion, nightly retrain and recalibration. Human effort is review, not operation. |
| Scaling | Configuration only | A new region is a YAML file. No architectural rewrite for more regions, technologies or resolutions. |
| Engineering risk | Moderate | Archetype weighting is the genuine unknown; mitigated by a clear-sky fitting procedure. |

## 6.4  Regulatory and market fit

The timing is unusually favourable. CERC halved the permitted deviation bands effective 1 April 2026 and is phasing the tolerance mechanism to zero by FY 2031. Forecast accuracy moved from an operational preference to a direct, escalating line item on every renewable generator’s P&L — in the current financial year. State commissions are moving in parallel.

Stated honestly: these regulations face active legal challenge, and the Karnataka High Court has stayed CERC’s revised mechanism. But the direction of travel — tighter tolerances, greater financial consequence for error — is consistent across CERC and multiple state commissions, and the curtailment problem exists regardless of settlement rules. The case does not rest on one regulation.

# 7   Impact and Benefits

Headline figures below are measured by third parties. Derived figures show their arithmetic and mark their assumptions, so any of them can be recomputed with a different input.

## 7.1  What better forecasting is worth — measured elsewhere

| Study | Finding |
| --- | --- |
| NREL, Western U.S. | At 24% wind penetration, a 20% improvement in forecast accuracy yielded USD 195 million in annual operating cost savings. |
| Berkeley Lab | NWP-based solar forecasts cost USD 1.00/MWh in errors, versus USD 1.50/MWh for persistence — a 33% reduction in error cost. |
| CENACE, Mexico | Upscaled forecasting across the national fleet would correct 622 MW/hr of deviation. |

## 7.2  Value model, with the arithmetic shown

| Step | Value | Basis |
| --- | --- | --- |
| India curtailment, FY 2025-26 | 2,100 GWh | measured |
| Share addressable by anticipation rather than physical limits | 25% | assumed |
| Recoverable with 24–72 h forecasting plus dispatchable storage | 40% | assumed |
| Recoverable energy | 210 GWh / yr | derived |
| Value at ₹3.00/kWh avoided cost | ≈ ₹63 crore / yr | derived |
| CO₂ avoided at 0.71 tCO₂/MWh | ≈ 149,000 t / yr | derived |

The 25% assumption is deliberately conservative: Ember attributes roughly 64% of Q1 2026 curtailment to transmission constraints, leaving a ~36% residual. We use less than that residual rather than more.

## 7.3  Benefits by stakeholder

| Stakeholder | Current pain | What they gain |
| --- | --- | --- |
| Grid operators / SLDCs | Scheduling conventional plant against uncertain renewable output | 24–72 h net-load outlook with calibrated bands; ramp alerts with six hours of lead time |
| Renewable generators | Deviation exposure at ±5% from April 2026 | Higher-accuracy schedules and anticipated curtailment windows |
| Energy traders | Position-taking without quantified uncertainty | P10/P50/P90 for position sizing and risk limits |
| Storage operators | Charge and discharge timing set by heuristics | Optimised dispatch against a 72-hour outlook |
| Planners and regulators | Where to site storage and transmission | Regional curtailment analytics and the battery sizing curve |

WHERE THESE CLAIMS STOP Forecasting does not build transmission lines, and Ember attributes most Indian curtailment to transmission constraints. We address the anticipation-driven remainder only. The reserve-reduction benefit is the softest figure we publish, because it requires an operator to change institutional practice on the strength of a calibration plot — that is slow, and we do not pretend otherwise. Our accuracy claim is validated on Belgian metered data; Indian results demonstrate transferability, not measured accuracy.

# 8   Risks, Limitations and Roadmap

Figure 6  Build sequence. Each phase ends at a demonstrable checkpoint — a phase that ends with “the refactor is nearly done” is where long projects die.

## Principal risks

| Risk | Severity | Response |
| --- | --- | --- |
| Train/serve skew — training on reanalysis, serving on forecasts | High | Same source and variable names both sides; a 7-day bias correction fitted on live forecasts; conformal recalibration on live errors. Residual skew remains — this is our largest honest limitation. |
| Curtailment unflagged in source data | Medium | Heuristic detection where output flat-lines below the physics estimate under good conditions; excluded from training. |
| Archetype weights fail to converge | Medium | Fall back to a single average archetype. Accuracy degrades; the pipeline stands. We report the degradation. |
| Cannot match the operator’s own forecast | Low | Never claimed. The target is to beat persistence and close the gap to a stated margin using only open data. |
| Live API failure during demonstration | Low | A replay mode serves a cached week of forecast runs and all model artifacts. We build the live path and present from cache. |

## What would change our mind

Each of these is a measurable outcome of the evaluation harness, not a matter of opinion. Building that harness in phase 0 is what makes it possible to be wrong in public and say so.

- If the physics baseline does not beat persistence at 24–72 hours, the physics-first thesis is wrong for this horizon.

- If the residual model does not beat a direct model on the raw target, residual learning adds nothing here and we simplify.

- If archetype weighting does not beat a single average archetype, the regional physics refinement is over-engineering and we remove it.

- If conformal intervals need more than half of capacity width, the problem may be too uncertain at 72 hours for reserve sizing, and we reduce the advertised horizon.

# 9   Research and References

## Datasets

| Source | Contents and access |
| --- | --- |
| Elia Open Data Portal | ods031/032 wind, ods086/087 solar, ods001/002/003 load. Measured generation, monitored capacity, operator day-ahead and week-ahead forecasts with P10/P90, and demand — all at 15 minutes, 2013 onward. Free, no API key. opendata.elia.be |
| Open-Meteo | Forecast and Historical APIs. Shortwave, direct-normal and diffuse radiation, global tilted irradiance, cloud cover by level, wind at 10/80/120/180 m, gusts. ECMWF, ICON, GFS and ensemble endpoints. No key for non-commercial use. open-meteo.com |
| Historical and modelled renewable energy production for India | University of Bristol. Hourly wind and solar capacity factors 1979–2022, state-level and 1°×1° gridded, installed capacity by state, georeferenced plant locations. CSV, NetCDF, GeoJSON. Zenodo DOI 10.5281/zenodo.7824872 |
| Open Power System Data | 35 European countries, generation, capacity and load; Germany resolved to four TSO zones. Used as a cross-check region. data.open-power-system-data.org |
| SDWPF (KDD Cup 2022) | Baidu / Longyuan. 134 turbines, 4.7 million records, 10-minute interval, with turbine x/y coordinates enabling wake and graph modelling. arXiv:2208.04360 |
| GEFCom2014 | The reference academic benchmark. Wind: 10 farms with ECMWF u/v at 10 m and 100 m. Solar: 3 plants with 12 ECMWF variables. Evaluated by pinball loss over 99 quantiles. Won by a generalized additive tree ensemble. |

## Methodology, standards and regulation

| Source | What it establishes |
| --- | --- |
| GET.transform (2024), International Best Practices in Solar and Wind Power Forecasting | Accuracy benchmarks by country and context; error grows nearly linearly with horizon; ~80% of world TSOs use centralised forecasting; optimal NWP combination is 5–6 models; quantified operating-cost benefits (NREL, Berkeley Lab, CENACE). |
| Development of a day-ahead solar power forecasting model chain for a 250 MW PV park in India (2023) | ECMWF and NCMRWF over India carry systematic positive irradiance bias, correctable to near zero with 20–25 days of training. A linear combination of two NWP models outperformed either alone. |
| Conformal prediction literature, 2025–2026 | Climate-invariant conformal intervals for multi-horizon solar and wind; adaptive conformal inference for PV; conformalised quantile regression — the two-stage pattern used here. |
| IEC 61400-12; Ineichen–Perez; SAPM; Perez transposition; NREL SPA | The physical models underlying the feature engine, implemented via pvlib-python and windpowerlib. |
| CERC Deviation Settlement Mechanism, notified for 1 April 2026 | Solar and hybrid bands ±10% → ±5%; wind ±15% → ±10%. X factor phased 100% to 0% by FY 2031. Cited alongside the Karnataka High Court stay and industry objections. |
| Ember — India curtailment analysis, 2026 | 2.1 TWh curtailed FY 2025-26 (1.3% of renewable generation); 300 GWh of ~470 GWh in Q1 2026 from transmission constraints; 34 GWh lost on 30 March 2026; ~10 GWh of storage needed; coal cycling to ~55% of rated capacity at midday. |

Citation practice — every external number carries its source and date; every derived number shows its arithmetic and marks its assumptions; contested sources are cited alongside the challenge, and limitations are stated before a reader finds them.